MSRS Report Format Documentation

Non-Firm Point-to-Point Transmission Service Resale Credits

Version 2

Revision History

| Date | Revision | Description |
| --- | --- | --- |
| 1/14/2009 | 1 | Initial Distribution |
| 2/21/2020 | 2 | Removed all references to eSchedule(s) and/or eSchedule(s) reports. |

# Report

MSRS Report Name: Non-Firm Point-to-Point Transmission Service Resale Credits

Report short name for User Interface: Non-Firm PTP Resale Credits

Download File Name Abbreviation: NFPTPResCr

Data Granularity: Hourly

Frequency: Updated Daily

Range Displayed on Report: Start Date through End Date

# Supported Billing Line Items

- Non-Firm Point-to-Point Transmission Service Resale Credit (2143)

# Report Content Summary

This displays the customer account’s hourly Non-Firm Point-to-Point Transmission Service Resale Credit for each resale non-firm transmission service reservation that is Confirmed and has Capacity greater than 0.

# Summary of Changes and Special Logic

- The Date, OASIS ID, Reservation Type, Time Block, Point of Receipt, Point of Delivery and Buyer will be listed as columns in the report; therefore column headers will not repeat in this report when several days and transmission reservations are downloaded in one file.

- Each daily instance of a transmission reservation will be represented by four separate, consecutive rows on this report.

- The Data Label column on the report identifies which types of information each row displays (Hourly Non-Firm PTP Resale Rate, Profile Capacity, Billable Profile Capacity or Non-Firm PTP Transmission Service Resale Credit).

- The first row will display the reservation’s Hourly Non-Firm PTP Resale Rate for each hour. The second row will display the reservation’s Profile Capacity for each hour.  The third row will display the reservation’s Billable Profile Capacity for each hour.  The fourth row will display the reservation’s Non-Firm PTP Transmission Service Resale Credit for each hour.  For each of these four rows, the values in the OASIS ID, Reservation Type, Time Block, Point of Receipt, Point of Delivery and Buyer columns will be identical.

- The Reservation Type column on the report identifies the capacity type of the transmission reservation.

- The Non-Firm PTP Transmission Service Daily Resale Credit for transmission reservations subject to Regional Through and Out Rate (RTOR) elimination will be set to zero.

- Daylight Saving Time (DST) Handling

- For online versions:

  - The online version of the report will display the number of hours in each day.  24 hours will appear for all days of the year except for DST switch days.  On the Spring DST switch day, only 23 hours will appear on the report (EPT hour ending 3 is skipped); on the Fall DST switch day, 25 hours will appear on the report (EPT hour ending 02 appears twice – the second hour ending 02 will be labeled 02* and come after the first hour ending 02).

- For download versions:

  - A 25th hour column (a second EPT Hour Ending 2) will appear in the report for all days, regardless of whether it is a DST switch day or not

    - This additional column will be placed immediately after the first hour ending 2

    - It will be name ‘EPT HE 02*’ for the CSV downloads and ‘EPT_HE_02X’ for the XML downloads

  - All non-DST switch days will display ‘0’ values in the second EPT HE 2 column

  - The Fall DST switch will be handled by populating the second HE 2 column

  - The Spring DST switch will be handled by displaying ‘0’ values in the second HE 2 and the HE 3 columns

# Report Columns

The following columns will appear in the body of the report:

| Online and CSV Column Name | XML Column Name | Column Number | Data Type |
| --- | --- | --- | --- |
| Customer ID | CUSTOMER_ID | 4000.01 | INTEGER |
| Customer Code | CUSTOMER_CODE | 4000.02 | VARCHAR2(6) |
| Date | DATE | 4000.04 | DATE (MM/DD/YYYY in online and CSV formats, YYYY-MM-DD in XML format) |
| OASIS ID | OASIS_ID | 4000.14 | NUMBER(*,0) |
| Parent OASIS ID | PARENT_OASIS_ID | 4000.54 | NUMBER(*,0) |
| Reservation Type | RESERVATION_TYPE | 4000.15 | VARCHAR2(30) See possible values below |
| Time Block | TIME_BLOCK | 4000.51 | VARCHAR2(10) See possible values below |
| Point of Receipt | POINT_OF_RECEIPT | 4000.52 | VARCHAR2(12) |
| Point of Delivery | POINT_OF_DELIVERY | 4000.53 | VARCHAR2(12) |
| Buyer | BUYER | 4000.17 | VARCHAR2(6) |
| Data Label | DATA_LABEL | 1140.11 | VARCHAR2(60) See possible values below |
| EPT HE 01 – EPT HE 24 - HE 02* | EPT_HE_01 EPT_HE_02 EPT_HE_02X EPT_HE_03 … EPT_HE_24 | 3013.01 - 3013.25 (HE 02* - 3013.25) | NUMBER |
| Version | VERSION | 4000.07 | VARCHAR2(12) |

Predetermined Data Label Values: Hourly Non-Firm PTP Resale Rate ($/MW per hour), Profile Capacity (MW), Billable Profile Capacity (MW), Non-Firm PTP Transmission Service Resale Credit ($)

Possible Reservation Type Values: NON_FIRM, NON_FIRM_INTERNAL, NON_FIRM_OFF_PEAK, NON_FIRM_ON_PEAK

Possible Time Block Values: day, hour, week, month

# CSV Report Example

See Excel file titled “Non-Firm Point-to-Point Transmission Service Resale Credits CSV Format.csv”

Note: This CSV file includes enhanced detail to illustrate column population for each transaction type.

# XML Report Example

See XML file titled “Non-Firm Point-to-Point Transmission Service Resale Credits XML Format.xml”

# Hyperlinks

The online version of this report does not contain hyperlinks.

# Supporting Calculations

Non-Firm PTP Transmission Service Resale Credits = (Hourly Non-Firm PTP Resale Rate * Profile Capacity)