MSRS Report Format Documentation

Non-Performance Assessment Billing Month Summary

Version 2

Revision History

| Date | Revision | Description |
| --- | --- | --- |
| 1/20/2020 | 1 | Initial Distribution |
| 3/30/2023 | 2 | Added new columns Total Import MW, Total Export MW, Net Import MW, Total PJM Monthly Bonus Holdback, Renamed column Total Available Bonus Performance Credits to Total Potential Bonus Performance Credits; Updated Supporting Calculations for Net Import MW and Bonus Performance Monthly Credit |

# Report

MSRS Report Name: Non-Performance Assessment Billing Month Summary

Report short name for User Interface:  NPA Billing Month Summary

Download File Name Abbreviation:  NPABMthSum

Data Granularity: Sub-hourly

Frequency: Updated monthly

Range Displayed on Report: Start Bill Month through End Bill Month

# Supported Billing Line Items

- Non-Performance Charge (1667)

- Bonus Performance Credit (2667)

# Report Content Summary

This report displays the customer account’s non-performance charge and/or bonus performance credit for the billing month where the monthly non-performance charge or the bonus performance monthly credit is non-zero.

# Summary of Changes and Special Cases

- Total Import MW, Total Export MW, and Net Import MW columns are only applicable during RTO wide events.

- Net Import MW column value is included in the Total Bonus Performance MW column value where applicable.

# Report Columns

The following columns will appear in the body of the report:

| Online and CSV Column Name | XML Column Name | Column Number | Data Type |
| --- | --- | --- | --- |
| Customer ID | CUSTOMER_ID | 4000.01 | INTEGER |
| Customer Code | CUSTOMER_CODE | 4000.02 | VARCHAR2(6) |
| Performance Assessment Interval Ending (EPT) | PA_INTERVAL_END_EPT | 4001.13 | VARCHAR2(40) mm/dd/yyyy HH24:MM format (Displays first interval of the day as hour 0 minute 05 and last interval of the day as hour 24 minute 00) |
| Performance Assessment Interval Ending (GMT) | PA_INTERVAL_END_GMT | 4001.14 | VARCHAR2(40) mm/dd/yyyy HH24:MM format (Displays first interval of the day in relation to EPT interval as hour 04 minute 05 or hour 05 minute 05 (EDT/EST depending) and last interval of the day as hour 04 minute 00 of the next day or hour 05 minute 00 of the next day (EDT/EST depending)) |
| Billing Month | BILLING_MONTH | 4000.03 | Date (Mon, YYYY) |
| Performance Assessment Area | PERFORMANCE_ASSESSMENT_AREA | 1667.12 | VARCHAR2(4000) |
| Total Non-Performance Charge ($) | TOT_NON_PERF_CH | 1667.54 | NUMBER(22,9) |
| Non-Performance Monthly Charge ($) | NON_PERF_MONTHLY_CH | 1667.01 | NUMBER(22,9) |
| Total Elapsed Non-Performance Monthly Charge ($) | TOT_ELAPSE_NON_PERF_CH | 1667.55 | NUMBER(22,9) |
| Total Import MW | TOTAL_IMPORT_MW | 2667.16 | NUMBER(22,9) |
| Total Export MW | TOTAL_EXPORT_MW | 2667.17 | NUMBER(22,9) |
| Net Import MW | NET_IMPORT_MW | 2667.18 | NUMBER(22,9) |
| Total Bonus Performance MW | TOT_BONUS_PERFORMANCE_MW | 2667.11 | NUMBER(22,9) |
| Total PJM Bonus Performance MW | TOT_PJM_BONUS_PERFORMANCE_MW | 2667.12 | NUMBER(22,9) |
| Total PJM Non-Performance Monthly Charge ($) | TOT_PJM_NON_PERF_MONTHLY_CH | 2667.13 | NUMBER(22,9) |
| Total PJM Monthly Bonus Holdback ($) | TOT_PJM_MONTHLY_BONUS_HOLD | 2667.19 | NUMBER(22,9) |
| Total Potential Bonus Performance Credits ($) | TOT_POT_BONUS_PERF_CR | 2667.14 | NUMBER(22,9) |
| Bonus Performance Monthly Credit ($) | BONUS_PERFORMANCE_MONTHLY_CR | 2667.01 | NUMBER(22,9) |
| Total Elapsed Bonus Performance Monthly Credit ($) | TOT_ELAPSE_BONUS_PERF_CR | 2667.15 | NUMBER(22,9) |
| Version | VERSION | 4000.07 | VARCHAR2(12) |

# CSV Report Example

See Excel file titled “Non-Performance Assessment Billing Month Summary CSV Format.csv”

# XML Report Example

See XML file titled “Non-Performance Assessment Billing Month Summary CSV Format.xml”

# Supporting Calculations

Net Import MW = Max(Total Import MW – Total Export MW , 0)

2667.18 = Max(2667.16 – 2667.17, 0)

Bonus Performance Monthly Credit = (Total Bonus Performance MW / Total PJM Bonus Performance MW) * (Total PJM Non-Performance Monthly Charge – Total PJM Monthly Bonus Holdback)

2667.01 = (2667.11 / 2667.12) * (2667.13 – 2667.19)