MSRS Report Format Documentation

Regulation Summary

Version 6

Revision History

| DATE | Revision | Description |
| --- | --- | --- |
| 10/19/2007 | 1 | Initial Distribution |
| 01/10/2011 | 2 | Corrected Regulation Obligation calculation in Supporting Calculations section |
| 8/20/2012 | 3 | Updated report to include the following columns: RMCCP ($/MWh) RMPCP ($/MWh) RMCCP Charge ($) RMPCP Charge ($) RMCCP Credit ($) RMPCP Credit ($) |
| 10/16/2013 | 4 | Updated report to include the following columns: Total Mileage Reg Adder (MWh) Total PJM Adjusted Reg Obligation (MWh) Mileage Ratio Adder (MWh) |
| 10/28/2014 | 5 | Updated RMCCP Credit and RMPCP Credit supporting calculations, and added details for PJM-Assigned Reg (MWh) under the Report Content Summary section. |
| 2/21/2020 | 6 | Removed all references to eSchedule(s) and/or eSchedule(s) reports. |

# Report

MSRS Report Name:  Regulation Summary

Report short name for User Interface: Regulation Summary

Download File Name Abbreviation: RegChCr

Data Granularity:  Hourly

Frequency:  Updated daily

Range Displayed on Report:  Start Date through End Date

# Supported Billing Line Items

- Regulation and Frequency Response Charge (1340)

- Regulation and Frequency Response Service Credit  (2340)

# Report Content Summary

This report displays the customer account’s daily Regulation Market Capability Clearing Price Charges and Credits, Regulation Market Performance Clearing Price Charges and Credits and Regulation Lost Opportunity Cost Charges and Credits for the RTO Regulation Zone where the account’s Regulation Obligation, RMCCP Credit, RMPCP Credit or Regulation Lost Opportunity Cost Credit is not equal to 0.

The credits summarized on this report are reflective of credits received by the customer account’s generation units and demand side resources that participate in the regulation market.

The PJM-Assigned Reg (MWh) column should only be used to support the Schedule 9-4: Regulation and Frequency Response data found on the Schedule 9 and 10 Charge Details report.  The Regulation Credits report supplies the customer with the supporting details for both the RMCCP Credit ($) and RMPCP Credit ($) values displayed on this report.

# Summary of Changes and Special Logic

- The date range total row will only appear in the online version of the report.  It will not appear in the CSV and XML versions of the report.

- The Report will now display EPT and GMT Hour Ending.

- Regulation Zone will no longer appear on the report as all charges and credits will apply to the RTO regulation zone.

- Real Time Load Ratio will no longer appear on the report.  In its place, the components of the RT load ratio, RT Load and Total PJM RT Load, will appear.

- Total Assigned Reg, Adjusted Reg Obligation and Total PJM Reg Lost Opportunity Cost Credit will also appear on the report.

# Report Columns

The following columns will appear in the body of the report:

| Online and CSV Column Name | XML Column Name | Column Number | Data Type |
| --- | --- | --- | --- |
| Customer ID | CUSTOMER_ID | 4000.01 | INTEGER |
| Customer Code | CUSTOMER_CODE | 4000.02 | VARCHAR2(6) |
| EPT Hour Ending | EPT_HOUR_ENDING | 4000.05 | VARCHAR2(40) mm/dd/yyyy HH24 format (Displays first hour of the day as hour 1 and last hour of the day as hour 24) |
| GMT Hour Ending | GMT_HOUR_ENDING | 4000.06 | VARCHAR2(40) mm/dd/yyyy HH24 format (Displays first hour of the day as hour 1 and last hour of the day as hour 00 of the following day) |
| Total Assigned Reg (MWh) | TOTAL_ASSIGNED_REG | 1340.18 | NUMBER(22,3) |
| Total Mileage Reg Adder (MWh) | TOTAL_MILEAGE_REG_ADDER | 1340.21 | NUMBER(22,3) |
| RT Load (MWh) | RT_LOAD | 1340.19 | NUMBER |
| Total PJM RT Load (MWh) | TOTAL_REG_PJM_RT_LOAD | 1340.20 | NUMBER |
| Reg Obligation (MWh) | REG_OBLIGATION | 1340.11 | NUMBER(22,3) |
| Bilateral Reg Sales (MWh) | BILATERAL_REG_SALES | 1340.12 | NUMBER |
| Bilateral Reg Purchases (MWh) | BILATERAL_REG_PURCHASES | 1340.13 | NUMBER |
| Adjusted Reg Obligation (MWh) | ADJUSTED_REG_OBLIGATION | 1340.14 | NUMBER(22,3) |
| Total PJM Adjusted Reg Obligation (MWh) | TOTAL_ADJ_REG_OBLIGATION | 1340.22 | NUMBER(22,3) |
| Mileage Ratio Adder (MWh) | MILEAGE_RATIO_ADDER | 1340.23 | NUMBER(22,3) |
| RMCCP ($/MWh) | RMCCP | 3001.44 | NUMBER(10,2) |
| RMPCP ($/MWh) | RMPCP | 3001.45 | NUMBER(10,2) |
| RMCCP Charge ($) | REG_RMCCP_CH | 1340.03 | NUMBER(22,2) |
| RMPCP Charge ($) | REG_RMPCP_CH | 1340.04 | NUMBER(22,2) |
| PJM-Assigned Reg (MWh) | PJM_ASSIGNED_REG | 2340.13 | NUMBER(22,3) |
| Self-Scheduled Reg (MWh) | SELF_SCHEDULED_REG | 2340.14 | NUMBER(22,3) |
| Reg Purchases (MWh) | REG_PURCHASE | 1340.15 | NUMBER(22,3) |
| Total PJM Reg Purchase (MWh) | TOTAL_PJM_REG_PURCHASE | 1340.16 | NUMBER(22,3) |
| Total PJM Reg Lost Opportunity Credit ($) | TOTAL_PJM_REG_LOC_CREDIT | 1340.17 | NUMBER(22,2) |
| Reg Lost Opportunity Cost Charge ($) | REG_LOC_CHARGE | 1340.02 | NUMBER(22,2) |
| RMCCP Credit ($) | RMCCP_CREDIT | 2340.32 | NUMBER(22,2) |
| RMPCP Credit ($) | RMPCP_CREDIT | 2340.33 | NUMBER(22,2) |
| Reg Lost Opportunity Cost Credit ($) | REG_LOC_CREDIT | 2340.16 | NUMBER(22,2) |
| Version | VERSION | 4000.07 | VARCHAR2(12) |

# CSV Report Example

See Excel file titled “Regulation Summary CSV Format.csv”

# XML Report Example

See XML file titled “Regulation Summary XML Format.xml”

# Hyperlinks

This report contains the following hyperlinks:

| Column Name | Hyperlinked Report |
| --- | --- |
| PJM-Assigned Reg (MWh) | Reg Credits report |
| Self-Scheduled Reg (MWh) | Reg Credits report |
| RMCCP Credit ($) | Reg Credits report |
| RMPCP Credit ($) | Reg Credits report |
| Reg Lost Opportunity Cost Credit ($) | Reg Credits report |

# Supporting Calculations

Effective Prior to 10/1/2012:

Adjusted Reg Obligation (1340.14) = Reg Obligation (1340.11) + Bilateral Reg Sales (1340.12) - Bilateral Reg Purchases (1340.13)

RMCCP Charge (1340.03) = Adjusted Reg Obligation (1340.14) * RMCCP (3001.44)

RMPCP Charge (1340.04) = Adjusted Reg Obligation (1340.14) * RMPCP (3001.45)

Reg Purchases (1340.15) = MAX (Adjusted Reg Obligation (1340.14) - Self-Scheduled Reg (2340.14),0)

Reg Lost Opportunity Cost Charge (1340.02) = Total PJM Reg Lost Opportunity Cost Credit (1340.17) * (Reg Purchases (1340.15) / Total PJM Reg Purchase (1340.16))

RMCCP Credit (2340.32) = See Regulation Credits Report documentation

RMPCP Credit (2340.33) = See Regulation Credits Report documentation

11/7/2013 Implementation date, effective retroactive to 10/1/2012:

Adjusted Reg Obligation (1340.14) = Reg Obligation (1340.11) + Bilateral Reg Sales (1340.12) - Bilateral Reg Purchases (1340.13)

Mileage Ratio Adder (1340.23) = Total Mileage Reg Adder (1340.21) * (Adjusted Reg Obligation (1340.14) / Total PJM Adjusted Reg Obligation (MWh) (1340.22))

RMCCP Charge (1340.03) = Adjusted Reg Obligation (1340.14) * RMCCP (3001.44)

RMPCP Charge (1340.04) = (Adjusted Reg Obligation (1340.14) + Mileage Ratio Adder (1340.23)) * RMPCP (3001.45)

Reg Purchases (1340.15) = MAX (Adjusted Reg Obligation (1340.14) - Self-Scheduled Reg (2340.14),0)

Reg Lost Opportunity Cost Charge (1340.02) = Total PJM Reg Lost Opportunity Cost Credit (1340.17) * (Reg Purchases (1340.15) / Total PJM Reg Purchase (1340.16))

RMCCP Credit (2340.32) = See Regulation Credits report documentation

RMPCP Credit (2340.33) = See Regulation Credits report documentation