MSRS Report Format Documentation

Synchronized Reserve Obligation Details

Version 7

Revision History

| Date | Revision | Description |
| --- | --- | --- |
| 10/19/2007 | 1 | Initial Distribution |
| 11/15/2007 | 2 | Clarified that the Total Subzone Obligation and the Total Subzone Load columns will display values that represent the Synchronized Reserve Zone total, rather than the subzone total, when their is NOT a split zone. The description previously incorrectly referenced that these columns will display total Synchronized Reserve Zone values when the prices are split. |
| 12/10/2007 | 3 | - Corrected the XML column name for the Total Subzone Load column. It was previously listed as ‘TOTAL_SUBZONE’ rather than ‘TOTAL_SUBZONE_LOAD’. - Corrected the XML column name for the Above Obligation Tier 1 Adjustment column. It was previously listed as ‘TIER1_ALLOC_TO_OBL’ rather than ‘ABOVE_OBL_TIER1_ADJ’. These changes affect the report description only. The sample XML files that were originally posted contain the correct column names. |
| 1/14/2008 | 4 | - Clarified that when the price is split between sub zones, Total Subzone Tier 1 Excess will display by subzone. When the price is not split between sub zones, Total Subzone Tier 1 Excess will display by Synchronized Reserve Zone. This column was not previously mentioned in the list of columns that receive this treatment. - Updated the calculation Tier 1 Allocation to Obligation. The documentation of the calculation for this column no longer multiplies Obligation Ratio Share of Excess Tier 1 by Total Subzone Tier 1 Excess. Instead, it now shows the calculation uses the minimum of Remaining Bilateral Adjusted Obligation or Obligation Ratio Share of Excess Tier 1, plus the minimum of Adjusted Synch Reserve Obligation or Tier 1 Estimate MWh. |
| 6/17/2008 | 5 | - Added new Total Subzone Remaining Bilateral Adjusted Obligation column - Updated calculation for Obligation Ratio Share of Excess Tier 1 to use Remaining Bilateral Adjusted Obligation rather than Synch Reserve Obligation |
| 9/10/2012 | 6 | Updated ‘Predetermined Synch Reserve Zone and Subzone combinations’ section to report design documentation. |
| 2/21/2020 | 7 | Removed all references to eSchedule(s) and/or eSchedule(s) reports. |

# Report

MSRS Report Name: Synchronized Reserve Obligation Details

Report short name for User Interface: Synchronized Reserve Obligation Details

Download File Name Abbreviation: SROblDet

Data Granularity: Hourly

Frequency: Updated daily

Range Displayed on Report: Start Date through End Date

# Supported Billing Line Items

- Synchronized Reserve Charge (1360)

# Report Content Summary

This report displays the customer account’s hourly data supporting the Above Obligation Tier 1 Adjustment, Tier 1 Allocation to Obligation and Synch Reserve Purchases values that are used in the calculation of the account’s Synchronized reserve charges (SRMCP charges, opportunity cost charges cleared and Tier 1 charges).  Data will display on this report for each hour where the account’s Synch Obligation is not equal to 0.

# Summary of Changes and Special Logic

- The date range total row will only appear in the online version of the report.  It will not appear in the CSV and XML versions of the report.

- The Report will now display EPT and GMT Hour Ending.

- Total Subzone Obligation, Remaining Bilateral Adjusted Obligation, Total Subzone Remaining Bilateral Adjusted Obligation, Total Subzone Tier 1 Excess and Obligation Ratio Share of Excess Tier 1 are displayed on the MSRS version of this report.

- Real Time Load Ratio will not appear on the report.  In its place, the components of the RT load ratio, Subzone Load and Total Subzone Load, will appear.

- This report will display data at a subzone level for all Synchronized Reserve Zones.

- If there is not a split zone (the price for subzones within a synchronized reserve zone are equal) the Total Subzone Obligation, Total Subzone Load, Total Subzone Remaining Bilateral Adjusted Obligation and Total Subzone Tier 1 Excess columns will display values for the whole Synchronized Reserve Zone total, rather than the Subzone total.

# Report Columns

The following columns will appear in the body of the report:

| Online and CSV Column Name | XML Column Name | Column Number | Data Type |
| --- | --- | --- | --- |
| Customer ID | CUSTOMER_ID | 4000.01 | INTEGER |
| Customer Code | CUSTOMER_CODE | 4000.02 | VARCHAR2(6) |
| EPT Hour Ending | EPT_HOUR_ENDING | 4000.05 | VARCHAR2(40) mm/dd/yyyy HH24 format (Displays first hour of the day as hour 1 and last hour of the day as hour 24) |
| GMT Hour Ending | GMT_HOUR_ENDING | 4000.06 | VARCHAR2(40) mm/dd/yyyy HH24 format (Displays first hour of the day as hour 1 and last hour of the day as hour 00 of the following day) |
| Synch Reserve Zone | SYNCH_RES_ZONE | 4000.33 | VARCHAR2(50) See possible values below |
| Subzone | SUBZONE | 4000.34 | VARCHAR2(50) See possible values below |
| Total Subzone Obligation (MWh) | TOTAL_SUBZONE_OBL | 1360.21 | NUMBER |
| Subzone Load (MWh) | SUBZONE_LOAD | 1360.22 | NUMBER(16,6) |
| Total Subzone Load (MWh) | TOTAL_SUBZONE_LOAD | 1360.23 | NUMBER(16,6) |
| Tier 2 Shortfall (MWh) | TIER2_SHORTFALL | 1360.32 | NUMBER(11,3) |
| Synch Reserve Obligation (MWh) | SYNCH_RES_OBL | 1360.24 | NUMBER |
| Bilateral Synch Reserve Sales (MWh) | BILAT_SYNCH_RES_SALES | 1360.25 | NUMBER |
| Bilateral Synch Reserve Purchases (MWh) | BILAT_SYNCH_RES_PURCHASES | 1360.26 | NUMBER |
| Adjusted Synch Reserve Obligation (MWh) | ADJ_SYNCH_RES_OBL | 1360.27 | NUMBER |
| Tier 1 Estimate MWh | TIER1_ESTIMATE_MWH | 1360.28 | NUMBER |
| Remaining Bilateral Adjusted Obligation (MWh) | REMAINING_BILAT_ADJ_OBL | 1360.29 | NUMBER |
| Total Subzone Remaining Bilateral Adjusted Obligation (MWh) | TOTAL_REMAINING_BILAT_ADJ_OBL | 1360.33 | NUMBER |
| Total Subzone Tier 1 Excess (MWh) | TOTAL_SUBZONE_TIER1_EXCESS | 1360.30 | NUMBER |
| Obligation Ratio Share of Excess Tier 1 (MWh) | OBL_RATIO_SHARE_EXCESS_TIER1 | 1360.31 | NUMBER |
| Tier 1 Allocation to Obligation (MWh) | TIER1_ALLOC_TO_OBL | 1360.12 | NUMBER |
| Above Obligation Tier 1 Adjustment (MWh) | ABOVE_OBL_TIER1_ADJ | 1360.14 | NUMBER |
| Tier 2 Self-Scheduled MWh | TIER2_SELF_SCHEDULED | 3000.63 | NUMBER |
| Synch Reserve Purchases (MWh) | SYNCH_RES_PURCHASES | 1360.15 | NUMBER |
| Version | VERSION | 4000.07 | VARCHAR2(12) |

# Prior to 10/1/2012:

# Predetermined Synch Reserve Zone and Subzone combinations:

| Synch Reserve Zone | Subzone |
| --- | --- |
| RFC | PJM Mid Atlantic |
| RFC | Non PJM Mid Atlantic |
| DOM | Dominion |

# Effective 10/1/2012:

# Predetermined Synch Reserve Zone and Subzone combinations:

| Synch Reserve Zone | Subzone |
| --- | --- |
| RTO | PJM Mid Atlantic Dominion (MAD) |
| RTO | Non PJM Mid Atlantic Dominion (MAD) |

# CSV Report Example

See Excel file titled “Synchronized Reserve Obligation Details CSV Format.csv”

# XML Report Example

See XML file titled “Synchronized Reserve Obligation Details XML Format.xml”

# Hyperlinks

This report does not contain hyperlinks.

# Supporting Calculations

Synch Reserve Obligation (1360.24) = Total Subzone Obligation (1360.21) * (Subzone Load (1360.22) / Total Subzone Load (1360.23)) + Tier 2 Shortfall (1360.32)

Adjusted Synch Reserve Obligation (1360.27) = Synch Reserve Obligation (1360.24) + Bilateral Synch Reserve Sales (1360.25) - Bilateral Synch Reserve Purchases (1360.26)

Remaining Bilateral Adjusted Obligation (1360.29) = MAX (Adjusted Synch Reserve Obligation (1360.27) - Tier 1 Estimate MWh (1360.28),0)

Obligation Ratio Share of Excess Tier 1 (1360.31) = Total Subzone Tier 1 Excess (1360.30) * (Remaining Bilateral Adjusted Obligation (1360.29) / Total Subzone Remaining Bilateral Adjusted Obligation (1360.33))

Tier 1 Allocation to Obligation (1360.12) = MIN(Remaining Bilateral Adjusted Obligation (1360.29) , Obligation Ratio Share of Excess Tier 1 (1360.31)) + MIN(Adjusted Synch Reserve Obligation (1360.27) , Tier 1 Estimate MWh (1360.28))

Above Obligation Tier 1 Adjustment (1360.14) = Adjusted Synch Reserve Obligation (1360.27)  - Tier 1 Allocation to Obligation (1360.12)

Synch Reserve Purchases (1360.15) = MAX (Above Obligation Tier 1 Adjustment (1360.14) - Tier 2 Self-Scheduled MWh (3000.63),0)